MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 2/16/99									BLOCK BUSTER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-108   BLOCK BUSTER

TRADE NAME: (Hanger), (Round), (24 oz.) DEODORANT BLOCKS

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX % WT		OSHA PEL		ACGIH TLV		SEC 313

Paradichlorobenzene		106-46-7		99.75			75 ppm			10 ppm			Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  174°C, 345.2°F

VAPOR PRESSURE: 1.027 @ 25°C

VAPOR DENSITY: 5.1

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.245 - 1.250

SOLUBILITY IN WATER:  0.008%

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE and ODOR:  Crystalline solid, cherry and lemon fragrance

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  125°F (tag closed up)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): LEL: 2.5 	UEL: NA

EXTINGUISHING MEDIA: Water spray-foam, CO2, dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: Wear protective clothing and NIOSH/MSHA approved self-contained breathing apparatus where exposure to vapors is possible.

UNUSUAL FIRE & EXPLOSION HAZARDS: Burns to from toxic, corrosive hydrogen chloride, and possible traces of phosgene.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY:  Stable

Conditions To Avoid: Keep away from open flame and hot glowing surfaces.

CHEMICAL INCOMPATIBILITY: None known

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide, smoke, soot, hydrogen chloride and phosgene

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: NA

OXIDIZER:  NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Inhalation or Ingestion: Paradichlorobenzene, at concentrations well above permissible exposure limits, can cause depression of the central nervous system and has been know to cause liver damage to rats and rabbits. Eyes: Irritation caused by vapor. Skin: Irritation caused by vapor.

CHRONIC EFFECTS OF OVER EXPOSURE: Headaches, dizziness, eyes, nose and throat irritation, nausea, feeling of drunkenness and cardiac sensitization may be experienced by some individuals where the concentration of paradichlorobenzene is well above the TLV in confined, poorly ventilated areas.

CARGINOGENICITY: Inhalation studies indicate paradichlorobenzene to be without carcinogenic effects. Based on living evidence in living animals and an absence of human data, the EPA has placed paradichlorobenzene in group C carcinogen category, possible human carcinogen.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash affected area with soap and water.

EYES: Flush with plenty of water for at least 15 minutes. Contact physician.

INGESTION: Rinse mouth. DO NOT induce vomiting. Give lots of milk or water. Contact physician immediately.

INHALATION: Remove to fresh air. In all cases where pain in effected area persists, call a physician.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  If airborne limits may be exceeded, use NIOSH approved respirator.

VENTILATION: Local exhaust: Use in well ventilated areas.

PROTECTIVE CLOTHING: None

EYE PROTECTION: Do not rub eyes with contaminated hands.

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: None

WORK/HYGIENIC PRACTICES: Wash thoroughly with soap and water after coming in contact with product.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Scoop up and put in closed container. Keep material out of sewers or open waterways. Allow small amounts to evaporate naturally.

WASTE DISPOSAL METHOD: The material should be burned in an approved incinerator or disposed of in an approved waste dump in accordance with applicable Federal, State and Local regulations.

PRECAUTIONS IN HANDLING: Store away from heat and open flames in tightly closed containers. Avoid excessive inhalation and contact with skin and eyes. Do not wear contact lenses when handling.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.